APOLLO New System Installation and PostgreSQL Setup SOP Windows deployment procedure for Apollo Tyre Inspection

| Document field | Value |
| --- | --- |
| Document type | Standard Operating Procedure (SOP) |
| Applies to | New Windows Apollo workstation installation |
| Database | PostgreSQL - eyresqc_apollo |
| Schema | apollo |
| Application command | python GUI.py |
| Revision | 1.0 |
| Prepared date | 28 July 2026 |

|  | Important Use this procedure with the complete validated Apollo deployment package. Do not create only selected database tables manually; apply the full numbered migration sequence. |
| --- | --- |

# 1. Purpose and scope

This SOP gives the exact installation and validation flow for bringing up the Apollo Tyre Inspection application on a new Windows computer. It covers the Python environment, required Python packages, PostgreSQL server, application database, schema migrations, configuration, startup validation, and installation troubleshooting.

# 2. Installation flow at a glance

| Stage | Action | Successful output |
| --- | --- | --- |
| 1 | Prepare Windows and drivers | Administrator access, network and vendor SDKs ready |
| 2 | Copy Apollo project | Complete source, configuration and migrations present |
| 3 | Create Apollo Conda environment | Correct interpreter and Python packages available |
| 4 | Install PostgreSQL | Local server listening on 127.0.0.1:5432 |
| 5 | Create role and database | apollo_user and eyresqc_apollo created |
| 6 | Configure Apollo | Database credentials and schema match PostgreSQL |
| 7 | Apply migrations | All Apollo tables, indexes and triggers created |
| 8 | Validate and launch | Database checks pass and GUI starts |

# 3. Fixed deployment values

| Item | Required value |
| --- | --- |
| Project folder | C:\Apollo_patch_pos |
| Conda environment | Apollo |
| PostgreSQL host | 127.0.0.1 |
| PostgreSQL port | 5432 |
| Database | eyresqc_apollo |
| Application database user | apollo_user |
| Schema | apollo |
| Database password | Strong machine-specific password stored securely |

|  | Password rule The password assigned to the PostgreSQL role apollo_user and the password in the Apollo configuration must be exactly the same. Do not store the real production password in Git or in this document. |
| --- | --- |

# 4. Detailed installation procedure

| 1 | Prepare Windows, network and hardware software |
| --- | --- |

Install Windows updates and restart the computer.

Install the validated NVIDIA driver required by the Apollo PyTorch build.

Install the validated Lucid Arena SDK/driver used by the cameras.

Install the validated Teledyne DALSA Z-Trak software/SDK when laser capture is used.

Configure the NICs, VLANs, static IP addresses and jumbo-frame settings according to the approved machine network document.

Confirm that the PLC and each camera/laser IP can be reached before live commissioning.

|  | Version control Use the same validated vendor SDK, driver, CUDA and PLC-library versions as the working Apollo machine. Do not upgrade production dependencies during installation unless the new versions were tested. |
| --- | --- |

| 2 | Copy and verify the Apollo deployment package |
| --- | --- |

Copy the complete project to:

C:\Apollo_patch_pos

GUI.py and the complete src folder.

The approved .env and configuration files.

requirements.txt and, when supplied, requirements-postgres.txt or environment.yml.

Models, recipes, media and configuration folders required by the deployed SKUs.

The numbered PostgreSQL migration files.

apply_apollo_postgres_migrations.py or the approved project migration runner.

Verify migration files:

cd /d C:\Apollo_patch_pos dir /s /b 0??_*.sql

Required migration names include:

001_phase1_foundation.sql 002_sku_recipe_device_tables.sql 003_inspection_cycle_tables.sql 004_binary_asset_tables.sql 006_final_runtime_cutover.sql

| 3 | Install Anaconda/Miniconda and create the Apollo environment |
| --- | --- |

Install the approved 64-bit Anaconda or Miniconda distribution.

Open Anaconda Prompt as the Windows user that will run Apollo.

Create or restore the Apollo environment.

Preferred when environment.yml is supplied:

cd /d C:\Apollo_patch_pos conda env create -f environment.yml conda activate Apollo

Example only when no environment file is supplied:

conda create -n Apollo python=3.11 -y conda activate Apollo

|  | Python version The validated environment.yml or deployment handover is the source of truth. Use the example Python version only when no validated environment definition is available. |
| --- | --- |

Confirm the active interpreter:

where python python --version python -m pip --version

| 4 | Install Python dependencies |
| --- | --- |

Install the main project dependencies:

cd /d C:\Apollo_patch_pos conda activate Apollo python -m pip install --upgrade pip python -m pip install -r requirements.txt

Install PostgreSQL and PLC Python packages:

python -m pip install --upgrade "psycopg[binary,pool]" python -m pip install "python-snap7<3"

When requirements-postgres.txt is present, this may be used instead:

python -m pip install -r requirements-postgres.txt

Verify imports:

python -c "import psycopg; import psycopg_pool; print('psycopg OK:', psycopg.__version__)" python -c "import snap7; print('snap7 OK:', snap7.__version__)"

|  | Package names The import is snap7, but the pip package is python-snap7. Psycopg connection pooling is supplied by psycopg-pool, normally installed using psycopg[binary,pool]. |
| --- | --- |

| 5 | Install PostgreSQL on Windows |
| --- | --- |

Run the approved 64-bit PostgreSQL Windows installer.

Install PostgreSQL Server, pgAdmin 4 and Command Line Tools.

Keep the server port as 5432 unless the Apollo configuration was deliberately changed.

Set and securely record the postgres administrator password.

Complete installation and confirm that the PostgreSQL Windows service is running.

Check the service and listening port:

sc query type= service state= all ¦ findstr /I postgres netstat -ano ¦ findstr :5432

Start the service when required (adjust version number):

net start postgresql-x64-17

|  | Expected result PostgreSQL must be listening on 127.0.0.1 or 0.0.0.0 at port 5432 before Apollo can create its connection pool. |
| --- | --- |

| 6 | Create the Apollo PostgreSQL role, database and schema |
| --- | --- |

Open psql as the postgres administrator:

"C:\Program Files\PostgreSQL\17\bin\psql.exe" -h 127.0.0.1 -p 5432 -U postgres

At the postgres=# prompt, confirm existing roles and databases:

\du \l

For a new installation, create the application role and set its password:

CREATE ROLE apollo_user WITH LOGIN; \password apollo_user

Enter the deployment password twice when prompted. Then create the database expected by Apollo:

CREATE DATABASE eyresqc_apollo OWNER apollo_user; \c eyresqc_apollo

Create the application schema and permissions:

CREATE SCHEMA IF NOT EXISTS apollo AUTHORIZATION apollo_user; GRANT ALL PRIVILEGES ON DATABASE eyresqc_apollo TO apollo_user; GRANT ALL ON SCHEMA apollo TO apollo_user; GRANT ALL ON SCHEMA public TO apollo_user; ALTER ROLE apollo_user IN DATABASE eyresqc_apollo SET search_path TO apollo, public;

Verify and exit:

SELECT current_database(), current_user; \dn \q

|  | Database name must match Apollo connects to eyresqc_apollo. Creating a different database such as apollo will not solve the startup error unless the Apollo configuration is also deliberately changed. |
| --- | --- |

| 7 | Configure Apollo database settings |
| --- | --- |

Open the Apollo .env or database configuration file and ensure that it resolves to the following connection values:

| Setting | Value |
| --- | --- |
| Host | 127.0.0.1 |
| Port | 5432 |
| Database | eyresqc_apollo |
| User | apollo_user |
| Password | Same password set with \password apollo_user |
| Schema | apollo |

Typical separate-variable format:

POSTGRES_HOST=127.0.0.1 POSTGRES_PORT=5432 POSTGRES_DB=eyresqc_apollo POSTGRES_USER=apollo_user POSTGRES_PASSWORD=<YOUR_SECURE_PASSWORD> POSTGRES_SCHEMA=apollo

Typical URL format:

DATABASE_URL=postgresql://apollo_user:<YOUR_SECURE_PASSWORD>@127.0.0.1:5432/eyresqc_apollo

|  | Use existing variable names Apollo versions may use POSTGRES_URL, DATABASE_URL or separate POSTGRES_* variables. Keep the variable names already used by the source code and change only the values. Do not add multiple conflicting connection definitions. |
| --- | --- |

Find active database settings in the project:

cd /d C:\Apollo_patch_pos findstr /S /N /I /C:"eyresqc_apollo" /C:"DATABASE_URL" /C:"POSTGRES_" /C:"DB_NAME" *.env *.py *.ini *.json

Check Windows/Conda overrides:

set ¦ findstr /I "POSTGRES DATABASE_URL DB_NAME PGPASSWORD" conda env config vars list

| 8 | Test the database login before migrations |
| --- | --- |

Direct psql login test:

"C:\Program Files\PostgreSQL\17\bin\psql.exe" -h 127.0.0.1 -p 5432 -U apollo_user -d eyresqc_apollo

Inside psql:

SELECT current_database(), current_user; \q

Python test from the Apollo environment:

conda activate Apollo cd /d C:\Apollo_patch_pos python -c "import getpass,psycopg; p=getpass.getpass('DB password: '); c=psycopg.connect(host='127.0.0.1',port=5432,dbname='eyresqc_apollo',user='apollo_user',password=p,connect_timeout=5); print('CONNECTED:',c.execute('SELECT current_database(),current_user').fetchone()); c.close()"

Expected result:

CONNECTED: ('eyresqc_apollo', 'apollo_user')

| 9 | Apply the complete Apollo database migration sequence |
| --- | --- |

The database connection alone is not sufficient. The numbered migrations create the required application tables, including inspection_cycles and alarm_events, together with their dependencies, indexes, constraints and triggers.

Place the migration installer in the project root, then run:

conda activate Apollo cd /d C:\Apollo_patch_pos python apply_apollo_postgres_migrations.py

Enter the apollo_user password when prompted. The runner must apply all numbered SQL files in order and replace the {{schema}} placeholder with apollo.

Expected final verification:

inspection_cycles ¦ alarm_events -------------------------+-------------------- apollo.inspection_cycles ¦ apollo.alarm_events SUCCESS: Apollo PostgreSQL migrations were applied.

|  | Do not run raw template SQL blindly The migration files contain a {{schema}} placeholder. Use the approved migration runner, which safely renders the schema name before execution. |
| --- | --- |

| 10 | Verify required PostgreSQL objects |
| --- | --- |

Connect as apollo_user:

"C:\Program Files\PostgreSQL\17\bin\psql.exe" -h 127.0.0.1 -p 5432 -U apollo_user -d eyresqc_apollo

Run verification queries:

\dt apollo.* SELECT to_regclass('apollo.inspection_cycles'); SELECT to_regclass('apollo.alarm_events'); SELECT setting_key, setting_value FROM apollo.application_settings WHERE setting_key = 'postgres_phase';

Expected minimum result:

apollo.inspection_cycles apollo.alarm_events

\q

| 11 | Start Apollo and perform startup validation |
| --- | --- |

Launch:

conda activate Apollo cd /d C:\Apollo_patch_pos python GUI.py

Successful database startup log:

PostgreSQL pool opened url=postgresql://***@127.0.0.1:5432/eyresqc_apollo schema=apollo min=1 max=8

Open Alarm Center and confirm that no relation does not exist message appears.

Open Inspection History and confirm that the page loads without a PostgreSQL table error.

Confirm the TYRES INSPECTED counter can query the database.

Proceed with PLC, camera, laser and GPU hardware checks.

|  | Hardware status Messages such as No PLC client, Cameras not checked or Laser not checked are separate commissioning conditions. They do not indicate a PostgreSQL installation failure. |
| --- | --- |

| 12 | Complete machine commissioning and create a baseline backup |
| --- | --- |

Load the approved machine .env, camera profiles, laser profiles, recipes and AI models.

Run the hardware test page and verify PLC, cameras, laser, GPU, storage and application health.

Run one controlled test inspection and verify that the cycle appears in Inspection History.

Verify alarm open, acknowledgement and recovery operations.

Create a secure backup of the PostgreSQL database and the validated Apollo configuration.

Example database backup command:

"C:\Program Files\PostgreSQL\17\bin\pg_dump.exe" -h 127.0.0.1 -p 5432 -U apollo_user -d eyresqc_apollo -F c -f C:\Apollo_Backup\eyresqc_apollo_initial.backup

# 5. Installation verification checklist

| Check | Pass criteria | Status |
| --- | --- | --- |
| Project copy | C:\Apollo_patch_pos contains complete source and migrations | ☐ |
| Conda | Apollo environment activates and where python points to it | ☐ |
| Psycopg | psycopg and psycopg_pool imports succeed | ☐ |
| Snap7 | import snap7 succeeds | ☐ |
| PostgreSQL service | Service running and port 5432 listening | ☐ |
| Role | apollo_user exists and password login succeeds | ☐ |
| Database | eyresqc_apollo exists and is reachable | ☐ |
| Schema | apollo schema exists | ☐ |
| Migrations | inspection_cycles and alarm_events exist | ☐ |
| GUI | PostgreSQL pool opened and no missing-relation errors | ☐ |
| Hardware | PLC/cameras/laser/GPU validated | ☐ |
| Test cycle | Inspection saved and visible in history | ☐ |

# 6. Troubleshooting reference

| Error / symptom | Cause | Correct action |
| --- | --- | --- |
| no pq wrapper available | psycopg installed without a usable binary libpq wrapper | Run: python -m pip install --upgrade "psycopg[binary,pool]" |
| No module named psycopg_pool | Pool package missing | Install psycopg[binary,pool] or psycopg-pool |
| No module named snap7 | PLC Python package missing | Run: python -m pip install "python-snap7<3" |
| PoolClosed / initialization incomplete | PostgreSQL unreachable or authentication failed; the failed pool was then closed | Fix the server/credentials and restart Apollo completely |
| password authentication failed for apollo_user | Role password and Apollo config password do not match | Reset with \password apollo_user and update the Apollo config |
| role apollo_user is absent in \du | Application role was not created | CREATE ROLE apollo_user WITH LOGIN; then set password |
| database eyresqc_apollo does not exist | Wrong database was created or Apollo points to a missing database | Create eyresqc_apollo owned by apollo_user, or deliberately correct the config |
| relation apollo.alarm_events does not exist | Database exists but migrations were not applied | Run the full migration installer |
| relation apollo.inspection_cycles does not exist | Inspection migration was not applied | Run all numbered migrations in order, not only one SQL file |
| Connection timeout expired | Server stopped, wrong host/port, firewall or networking issue | Start PostgreSQL; verify 127.0.0.1:5432 and netstat output |

# 7. Rules for future installations

Use one approved deployment package and one approved environment definition.

Use eyresqc_apollo unless the application configuration is intentionally changed and tested.

Apply migrations every time a new database is created.

Never create substitute tables manually to silence errors.

Restart Apollo after changing database credentials because a closed pool is not reusable.

Keep real passwords outside source control and deployment documents.

Capture the final package versions, driver versions and database backup in the machine handover record.

# Appendix A - Quick command sequence

:: 1. Activate environment and install required Python packages conda activate Apollo cd /d C:\Apollo_patch_pos python -m pip install -r requirements.txt python -m pip install --upgrade "psycopg[binary,pool]" python -m pip install "python-snap7<3" :: 2. Open PostgreSQL administration console "C:\Program Files\PostgreSQL\17\bin\psql.exe" -h 127.0.0.1 -p 5432 -U postgres -- 3. Run inside psql CREATE ROLE apollo_user WITH LOGIN; \password apollo_user CREATE DATABASE eyresqc_apollo OWNER apollo_user; \c eyresqc_apollo CREATE SCHEMA IF NOT EXISTS apollo AUTHORIZATION apollo_user; GRANT ALL PRIVILEGES ON DATABASE eyresqc_apollo TO apollo_user; GRANT ALL ON SCHEMA apollo TO apollo_user; GRANT ALL ON SCHEMA public TO apollo_user; ALTER ROLE apollo_user IN DATABASE eyresqc_apollo SET search_path TO apollo, public; \q :: 4. Apply schema migrations python apply_apollo_postgres_migrations.py :: 5. Launch Apollo python GUI.py

|  | Final acceptance condition The installation is complete only when PostgreSQL pool opened is logged, the required apollo tables exist, Alarm Center and Inspection History load without database errors, and one controlled inspection cycle is stored successfully. |
| --- | --- |